LAN Topologies, Subnetting, ARP & DHCP

Table of Contents

Star Topology

Bus Topology

Ring Topology

Switch

Router

Subnetting

ARP

DHCP

## Star Topology

Definition: Devices connect to a central device (switch/hub) that forwards traffic to the target.

Key points:

Most common today due to reliability and scalability.

Central device forwards only where needed (with a switch) vs broadcast (hub).

Costs more: extra cabling + dedicated equipment.

Single point of failure at the central device.

Scaling increases maintenance and troubleshooting complexity.

Real-world scenario: Office floor with a 24/48‑port switch in a closet; if the switch dies, all connected PCs lose connectivity.

## Bus Topology

Definition: All devices share a single backbone cable; signals travel along the same shared medium.

Key points:

Easy and cost‑efficient to set up (minimal cabling/equipment).

Single backbone becomes a bottleneck under simultaneous traffic.

Difficult fault isolation because all traffic shares one path.

Single point of failure: if the backbone breaks, the segment is down.

Real-world scenario: Legacy lab with a coax backbone; one cable cut disrupts the entire segment.

## Ring Topology

Definition: Devices form a loop; data passes device‑to‑device in one direction (token-style).

Key points:

Low cabling and less reliance on dedicated central hardware.

Deterministic forwarding—each device relays if it has no data to send.

Simpler troubleshooting due to single direction of flow.

Inefficient pathing: frames may traverse many devices to reach the target.

Break (cable/device) typically brings the whole ring down.

Real-world scenario: Manufacturing line devices in a loop; a failed node or cut cable stops the ring.

## Switch

Definition: Layer‑2 device aggregating many ports; forwards frames to the correct port based on learned MAC addresses.

Key points:

Common in larger networks (schools, businesses): 4/8/16/24/32/64‑port models.

Reduces unnecessary traffic vs hubs by unicast forwarding.

Can be cascaded/interconnected; multiple paths improve redundancy.

Real-world scenario: School network core switch connecting labs and printers; a second uplink provides a backup path.

## Router

Definition: Connects different networks (subnets) and forwards packets between them using routing.

Key points:

Performs routing: selects paths across networks to deliver data.

Interconnecting routers/switches can add redundant paths (higher availability).

Real-world scenario: Home router links your LAN (192.168.1.0/24) to the ISP network; dual uplinks provide failover.

## Subnetting

Definition: Dividing a larger network into smaller sub‑networks to organize hosts and control broadcast domains.

Key points:

Uses a subnet mask (32 bits; 4 octets, 0–255 each).

Key addresses: Network (e.g., 192.168.1.0), Host (e.g., 192.168.1.100), Default Gateway (e.g., 192.168.1.254).

Benefits: efficiency, security, administrative control.

Real-world scenario: Cafe separates employee POS devices and public Wi‑Fi into different subnets while sharing the same Internet link.

## ARP

Definition: Resolves IP addresses to MAC addresses using ARP Requests and Replies; mappings cached locally.

Key points:

ARP Request: broadcast asking who has a given IP.

ARP Reply: owner of the IP responds with its MAC.

Entries stored in the ARP cache for future frames.

Real-world scenario: A PC ARPs to learn the router’s MAC (default gateway) before sending traffic off‑subnet.

## DHCP

Definition: Automatically assigns IP settings to devices using the DORA sequence (Discover, Offer, Request, ACK).

Key points:

Device broadcasts DHCP Discover to find a server.

Server replies with DHCP Offer including an IP lease.

Client sends DHCP Request to accept the offer.

Server finalizes with DHCP ACK; client can use the IP.

Real-world scenario: New phone joins office Wi‑Fi and receives an IP, gateway, and DNS automatically via DHCP.